2/6/2026 - Chris

Soldered Bullet Connectors to Board #1

Found Error in LDO - Voltage not dropping from 5V to 3.3V

Solution - Reverse Shockley Diode

2/6/2026 - Seth

Checking Crystal Oscillator frequency

Continuing CLI LoRa program

2/8/2026 - Chris & Deyis -2pm

Soldered on correctly the Schoktee diode on PCB1 & PCB2

Double checked the voltage coming in and out of the Buck and LDO were 5V and 3.3V.

Doubled check the voltage domains on the Barometer and IMU bere at 3.3 volts.

MAKE SURE THE SWITCH IS POSITIONED ON THE BOOT TO SHORT VIA GND, or else MCU may degrade!

2/13/2026 - Connor and Seth

Tried to connect JTAG ports to Fusion Dev Board/ST Link Programmer while plugged into power supply at 14.8V

Current Draw was higher than expected - Cause was probably a short on the board

2/18/2026

Tried to check current using power supply, current was about 300mA at 14.8V and 120mA at about 1.2V

Board # 2 Believed to be Dead

Soldered Bullet Connectors on U$11 and U$2, put shrink tubing on both bullets. All connections look good

2/24/26 —Christopher

- Board 1 & Board 2 may still be revivable, Board 3 is dead

- I believe its the Buck that is causing the power and VDD plane to short and cause cross talk on the layers.

- Buck was removed from Board 3, but is unrepairable due to copper trace exposure and solder contacts being damaged. To repair requires special equipment we cannot afford.

- We can run lower Voltage through the motors

- Overvoltage from the ESC is impossible since they have a Rubycon ZLH 470uF ultra-low ESR capacitor. It is specifically designed to handle high-frequency ripple current and voltage spikes.BUT it wont protect the buck, we would need to solder on an additional 1000uF capacitor at 25V to ensure nothing comes back.

- WE WILL FLY SLOW AND STEADY

- A TVS DIODE IS THE BEST WAY

- The PDB is the absolute perfect place to mount them. Instead of trying to awkwardly splice a diode into a wire or cram it onto the buck converter, you can solder the TVS diode and a nice big bulk capacitor directly across the main Positive and Negative pads of the PDB. This protects everything plugged into the board simultaneously (GEMINI)

2/25/26 — Christopher

- TVS diode has been selected just waiting on approval from tandon for Friday, a Littlefuse inc 5KP13A TVS diode, its a non surface mounted Transient Voltage Suppression diode that will start to eat away voltage at 14.4V to 15.9 V with an ABSOLUTE MAX at 21.5V

- We’re also going to be using a xt90-S connector to slowly bring up the voltage of our board instead of the initial surge of power that goes onto the board, this can cause a transient spike if we have high capacitance…which i assume we do, this is common with higher cell lipo batteries like the 4s that we have.

- Desoldered the LDO, Buck and the C2A and C3B capacitors.

- THIS IS JUST A THEORY!!! ^^^^

–Another theory is that we have no transient voltage from the battery, but will still need a TVS diode due to the ESC back peddling voltage into our power domain, so the TVS is still a good idea. THE SCHOKLEY DIODE should protect us from the voltage domains back pedaling, from the buck. IF THIS IS NOT ON THE BOARD CORRECTLY, WERE RUNNING RISKS of

- Looks like on board 3 we also have the LDO completely shot at well, its all shorted together once again on that power domain, Will remove the LDO right now

- The LDO is now known to be completely shot as well, so buck surge and LDO surge

2/27/26—Christopher

- Board 3 has an issue on the 3.3V domain when it comes to programming the STM32 on the Jtag connection

- Possible issues is the IMU being shorted due to the high influx of current, messing up the transistors and causing a short

- Confirmed by Connor and Chris.

- XT90 connector replaced by a XT90-S connector in order to limit the amount of current that surges through the board, will measure the transient voltages later. The connector has a 5.6ohm resistor.

3/2/2026– Christopher

- Found the short from 2/27 coming from the IMU and desoldered it off, there’s no longer a short and everything behaves as expected on the domains, further testing is required.

- JTAG MAY HAVE CAUSED THE SHORT, WE MUST KNOW WHY BEFORE PROCEEDING!!

- Measured if there’s any transient voltages on the new xt90-s Connection, There is none

3/11/2026 - Seth

Successfully outputted data from LoRa module PCB to CLI on PC

Implemented Debug mode on CLI

Released CLIv3.1

Will need to configure on how output sensor values to UART